STL-IP GPIOs

AudioTX STL-IP is able to send logic level signals alongside and in sync with your audio. This can be used for remote operation of equipment or relaying of signals between locations where STL-IP devices are installed. The unit has 4 inputs and 4 outputs which operate using 5 volt TTL logic.  The GPIO inputs and outputs can ONLY accept and provide logic signals - they will not sink or source current greater than 1 milli-Ampere (1ma).

DO NOT connect a device that causes a current flow of greater than 1mA as doing so will cause damage to the STLIP unit. Simple and cheap logic chips or transistor drive circuits must be used to buffer the inputs and outputs for, say, the connection of LEDs or relays.

Operation is straight forward. No additional setup or configuration is required.

- The TTL levels present on the STL-IP GPIO inputs are reproduced on GPIO outputs of the (or all) remote STL-IP units receiving audio from the local device.

- If the local STL-IP unit is receiving audio from a remote device, the states of the GPIO inputs at the remote device are reproduced on the GPIO outputs of the local STL-IP device. The pinouts of the GPIO connector are shown below:

### Pin Input/Output Function

| ## 13 In Input A 12 In Input B 10 In Input C 11 In Input D | 6 Out Output A 7 Out Output B 8 Out Output C 9 Out Output D |
| --- | --- |
| 18-21 - Ground / 0v | 22-25 - Ground / 0v |

# Status  outputs  available  on  the  GPIO  connector

The GPIO connector also carries 4 status outputs which show the operational status of the local STL-IP unit and 2 additional outputs show the status of the Audio Silence/Overload detectors. These outputs can be used to switch to backup audio feeds in case of network/mains failure. The Status outputs can ONLY provide logic signals - they will not sink or source current greater than 1 milli-Ampere (1ma).

DO NOT connect a device that causes a current flow of greater than 1mA as doing so will cause damage to the STL-IP unit

Pin Input/Output Function

| #### 2 Out | Power/Status ok. Equivalent to green power light on front panel. |
| --- | --- |
| 3 Out Error | When this signal is high, the other status outputs are invalid. For critical applications, for example if you plan to use the RX status output (below) to switch to/from backup feeds/audio, you should use logic that will only accept the other pins as valid when this pin is low and pin 2 (Power/Status ok) is high. |
| 4 Out | TX ok. Equivalent to green TX light on front panel. |
| 5 Out | RX ok. Equivalent to green RX light on front panel. |
| 16 Out | Audio input level overload detector |
| 17 Out | Silence detector triggered (input/output or both) |